Torque Specs

These specs may not be correct for your application.

Verify your engine specs with a service manual for your year and model.

always torque bolts in three equal increments

| Bolts or Parts | Lube or Sealer | Torque to: |
| --- | --- | --- |
| Main Caps | Engine Oil | 105 ft-lbs. |
| Warning!!! If you are using ARP bolts, you MUST use their specs. Click Here! |  |  |
| Connecting Rod Bolts 351C *Boss 351C | Engine Oil | 45 ft-lbs. *50 ft-lbs. |
| Cylinder Heads 351C *Boss 351C | Engine oil (blind hole) Sealer (water jacket) | 105 ft-lbs. *125 ft-lbs. |
| Rocker Arms | Engine Oil | 20 ft-lbs. |
| Oil Pump | Engine Oil | 25 ft-lbs. |
| Oil Pan | Engine Oil | 12 ft-lbs. |
| Camshaft Thrust Plate | Thread Locker | 12 ft-lbs. |
| Cam Bolt (upper gear) | Thread Locker | 40 ft-lbs. |
| Front Cover | Engine Oil | 20 ft-lbs. |
| Intake Manifold 5/16 Bolts (Cast Iron Heads) *3/8 Bolts | Non- Hardening Sealer | 25 ft-lbs. *30 ft-lbs. |
| Valve Cover | Engine Oil | 10 ft-lbs. |
| Flexplate (Automatic) Flywheel (Clutch) | Thread Locker | 85 ft-lbs. |
| Clutch Pressure Plate | Thread Locker | 35 ft-lbs. |
| Centerbolt (Harmonic Damper) | Thread Locker | 90 ft-lbs. |
| Bellhousing (Transmission to Block) |  |  |